ECD Ltd 	    	      Technical Services Document :- HT52155 Set-Up	 	         02-11-2001

PROGRAMMING THE HT52155 INTELLIGENT SERIAL MODULE

RS232 / Cash Drawer Interface

Note: - The HT52155 Intelligent CD Interface Is Supplied With Default Program

‘Esc p m n1 n2’ Loaded. If Using This EPSON Code To Open The Cash Drawer, There Is No Need To Re-Program HT52155.

# PROGRAMMING PROCEDURE

This example demonstrates the programming for Cash Drawer Opening Code  HEX (07)

Run rs232INS.exe from Windows; choose a temporary destination directory.

- The program will then automatically run SETUP.exe placing the necessary files into a default directory C:\RS232 (Icons being created on the Start menu file).

- Connect the HT52155 I.S.M. to the Cash Drawer (RJ12 Socket); the RS232 Port (DB9F Socket) & the 24Vdc Power Supply.

- Run the Program C:\RS232\…….

# In the ‘Communication Port Selection’ window :-

- Select COM Port to be used with the Interface e.g. ‘COM1’

- Activate the ‘Use Selected Port’ Button.

# In the ‘Intelligent Cash Drawer Interface - setup program’ :-

- Select the ‘Star/Citizen’ programming mode using the Scroll-Down Bar.

- Select the appropriate BAUD Rate (9600).

- Enter the appropriate ‘Cash Drawer Opening Code’ in HEX ( 07).

[Note :- The ‘Intelligent Cash Drawer Interface Set-Up Program’ Window may be extended downwards

(Mouse ‘click & drag’ bottom frame edge) to view ‘Local & Remote Data Code’.]

- Activate the ‘Program’ button to open the ‘Program_Interface’ window.

## In the ‘Program_Interface’ window :-

- Activate the ‘Program’ button.

- Press the ‘OK’ button in the ‘Power Down’ information window.

- Activate the ‘Quit’ button.

# Returning to the ‘Intelligent Cash Drawer Interface - setup program’ :-

- Check the programmed details are correct.

- Activate the ‘Open Drawer’ button to check satisfactory operation of the Cash Drawer.

- Activate the ‘Program’ button to return to the ‘Program_Interface’ window.

## In the ‘Program_Interface’ window :-

- If  required, activate the ‘Write Protect’ check box (To prevent the program code being overwritten).

- Activate the ‘Program’ button

- Press the ‘OK’ button in the ‘Power Down’ information window.

- Remove the 24Vdc Power Lead from the Module to ensure the Programmed Settings are ‘Fixed’.

The Interface Module is now ready for installation and should be Labelled with the appropriate programming details.

VFD-CB/TSD		17-01-2002